- Introduction

1.1 Mini Engineering Data Processor: Overview

This hybrid C++/Assembly program simulates a dual-purpose engineering unit:

- Sensor Analysis Module: Utilizes x87 FPU assembly for high-precision floating-point arithmetic to calculate statistical metrics (Min, Max, Avg, StandardDeviation) and perform temperature unit conversions.

- Image Processing Module: Uses MMX SIMD assembly to parallelize brightness adjustments on raw pixel data, employing saturated arithmetic to ensure data integrity.

1.2 Engineering Relevance

- Sensor Analysis (IoT & Industry): Critical for industrial controllers and automotive ECUs. Real-time statistical computation enables immediate fault detection (e.g., system overheating) and digital signal smoothing for noisy sensors.

- Image Brightness (Computer Vision): A fundamental step in autonomous driving and medical imaging. SIMD optimization is essential here, allowing the CPU to process multiple pixels simultaneously to meet the strict latency requirements of real-time video feeds.

- FPU Module Section

2.1 Temperature Data Description

The dataset simulates a series of temperature readings collected from an environmental sensor.

- Data Type: Double-precision floating-point numbers (double), each occupying 8 bytes to ensure high precision for scientific calculations.

- Sample Size: 10 sample points.

- Example Values: The dataset includes Celsius values such as 25.5, 26.0, 24.8, 27.2, and 23.5. These values represent a typical range of room temperature fluctuations, used to test the accuracy of the statistical analysis algorithms.

2.2 Algorithm Explanation

The CalcStats function leverages the x87 FPU (Floating Point Unit) register stack to efficiently calculate the average, minimum, and maximum values in a single pass.

- Initialization: The program loads the first element of the array and duplicates it three times onto the FPU stack. This initializes the "Maximum," "Minimum," and "Running Sum" registers with the first value, avoiding the need for arbitrary initial constants.

- Loop Processing & Comparison Logic: The program iterates through the remaining temperature values. For each value loaded into ST(0), it adds it to the running sum and then compares it against the current extremes.

The following assembly snippet illustrates exactly how the function determines if the current value should replace the current Maximum (Max) or Minimum (Min) without leaving the CPU registers:

; Update Max: Compare Current Val (ST0) vs Current Max (ST1)

fcomi st(0), st(1)      ; Compare floating-point values

jbe @CheckMin           ; If Val <= Max, jump to check Minimum

fst st(1)               ; If Val > Max, copy Val to Max register

; Update Min: Compare Current Val (ST0) vs CurrentMin (ST2)

@CheckMin:

fcomi st(0), st(2)      ; Compare Val vs Min

jae @NextVal            ; If Val >= Min, jump to next iteration

fst st(2)               ; If Val < Min, copy Val to Min register

@NextVal:

fstp st(0)              ; Pop Val Stack: Max, Min, Sum

add esi, 8

dec ecx

jnz @StatsLoop

Finalization: After processing all elements, the final Maximum and Minimum values are popped from the stack and stored in memory. The "Running Sum" is then divided by the total sample count to calculate the Average.

- MMX Module Section

3.1 Pixel Data Representation

In this module, the image data is represented as a contiguous array of 8-bit integers, where each byte corresponds to a single pixel's brightness value (assuming a grayscale image format for this demonstration). The values range from 0 (black) to 255 (white).

For example, a strip of 8 pixels might look like this in memory: [ 10, 55, 120, 200, 230, 250, 5, 100 ]

When processing an image with a resolution of, for instance, 640x480, the array would contain 307,200 such bytes. The MMX module is designed to iterate through this array, adjusting the brightness of every pixel by a fixed constant (e.g., adding brightness).

3.2 Packed Bytes and SIMD Efficiency

The core advantage of this module lies in its use of SIMD (Single Instruction, Multiple Data) technology via the MMX instruction set.

In a standard (scalar) approach, the CPU would load one pixel (1 byte), add the brightness value, store it back, and repeat this process for every single pixel. This requires a loop to execute hundreds of thousands of times for a standard image.

Packed Bytes allow us to treat a 64-bit MMX register (like MM0) not as one giant number, but as a container for eight separate 8-bit integers.

Why SIMD is faster:

- Parallelism: By loading 64 bits of data at once, we fit 8 pixels into a single register.

- Reduced Instruction Count: A single MMX instruction (e.g., PADDUSB) performs the addition on all 8 pixels simultaneously.

- Efficiency: This effectively reduces the number of addition instructions required by a factor of 8, significantly lowering the CPU overhead for fetching and decoding instructions compared to a standard loop.

3.3 Normal Addition vs. Saturated Addition

A critical challenge in image processing is handling arithmetic overflows.

- Normal Addition (Wraparound): In standard 8-bit arithmetic, if a result exceeds the maximum value of 255, it "wraps around" starting from 0.

  - Example: 250 + 20 = 270. In 8-bit binary, 270 is 100001110. The leading 1 is lost, leaving 00001110 (decimal 14).

  - Visual Effect: A very bright pixel (250) suddenly becomes very dark (14), causing ugly visual artifacts known as "inversion."

- Saturated Addition (Clamping): MMX provides specific instructions for saturation (e.g., PADDUSB - Packed Add Unsigned with Saturation Byte). If a result exceeds 255, it is "clamped" to 255. If it goes below 0, it is clamped to 0.

  - Example: 250 + 20 = 270 → Clamped to 255.

  - Visual Effect: The pixel stays white, preserving the image integrity.

3.4 Brightness Adjustment Data Table

The following table demonstrates the effect of increasing brightness by +50 on a sample set of pixels, comparing standard arithmetic with the MMX saturated arithmetic used in this module.

| Pixel Index | Original Value | Adjustment | Standard Add (Wrap) | MMX Saturated Add | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | +50 | 150 | 150 | Normal range |
| 2 | 50 | +50 | 100 | 100 | Normal range |
| 3 | 200 | +50 | 250 | 250 | Near limit |
| 4 | 230 | +50 | 24 (280 - 256) | 255 | Saturation applied |
| 5 | 250 | +50 | 44 (300 - 256) | 255 | Saturation applied |
| 6 | 0 | +50 | 50 | 50 | Normal range |
| 7 | 128 | +50 | 178 | 178 | Normal range |
| 8 | 210 | +50 | 4 (260 - 256) | 255 | Saturation applied |

As shown in pixels 4, 5, and 8, the MMX module correctly clamps values to 255, preventing the dark artifacts that would occur with standard addition.

- Conclusion & Teamwork

The primary challenge of this project was mastering the x87 FPU's rotating stack to ensure accurate statistical calculations without data corruption. This experience highlighted the distinct performance advantages of MMX SIMD parallel processing over traditional scalar operations for real-time image manipulation. Aqua acted as Lead Developer, implementing the core Assembly logic and software architecture, while Omen served as Technical Analyst, responsible for conducting experiments, verifying results, and authoring the final engineering report.